# Kestrel-2 Test Cell Integration Specification (DRAFT)

Author: Victor Nwosu

Date: 2026-06-26

DRAFT — work in progress, circulated for comment. Do not issue to customers. Several sections are TBD pending engineering input.

## 1. Mechanical docking

The handler docks to the tester manipulator on a three-point kinematic interface. Docking repeatability requirement is TBD pending the plunge force work; the current working figure of 50 um is inherited from the previous generation and has not been confirmed for Kestrel-2.

## 2. Thermal

| Parameter | Value | Confidence |
| --- | --- | --- |
| Temperature range at device | -45 C to +150 C | Confirmed |
| Soak to within 2 C | < 6 minutes | Confirmed |
| Site-to-site spread | 0.6 C measured | Measured at Tallgrass |
| Purge dew point requirement | -60 C at the site | Confirmed |
| House air requirement | None; membrane dryer on board | Confirmed |

## 3. Throughput

7,850 UPH measured on a customer device at four sites. The 8,000 UPH figure in marketing material is a target. TBD: whether the published specification should be changed to the measured number.

## 4. Changeover

Measured at 31 minutes by a trained engineer without the auto-teach routine. With auto-teach the expected figure is approximately 26 minutes. The datasheet currently claims under 25 minutes, which is not supportable. Placeholder: under 30 minutes pending a decision by Product.

## 5. Electrical and safety interfaces

Tester interlock loop: TBD, waiting on the tester vendor's document.

Dew point sensor interlock is safety rated with a hard stop, not a software warning.

Force sensor failure behaviour: TBD (see plunge force work).

## 6. Open questions

Contactor life claim: awaiting the 500,000 insertion life test.

Who owns the preventive maintenance schedule for the membrane dryer?

Does the customer or Meridian supply the changeover kit storage cart?